��������������������Ŀ

� TI92 LINK PROTOCOL �

����������������������

Release 1.0  03/21/1997 by

Vincent CARON (caron@ec-lyon.fr)

"How to communicate with the TI92 from a computer & PC application"

HARDWARE

��������

1 - Home made cable

2 - Hardware protocol

SOFTWARE

��������

1 - TI packets

2 - Packets description

3 - Variable data description

4 - Retrieve/send variable

5 - TI92 directory

6 - Screen dump

7 - Backups

8 - Remote control

PLEASE NOTE :

�������������

- This technical documentation file comes along the PLINK! communication

software. Please contact the author for more informations.

- All numbers are in HEXADECIMAL format (OFFSETs and DATA BYTEs), except

when stated. Example : 1..27d reads "1 to 27 decimal".

- All strings are in C format. They are arrays [1..maxsize] of char, with

an ending zero if the actual size of the string is less than maxsize.

In this case, any char after a #0 has no meaning and is not obviously

equal to #0.

It seems that the TI92 pads strings with #0 all the time.

- Data description convention :

offset     length (bytes)     description/type

+0           1                  beginning (byte [unsigned])

+xx          yy                 ...

You will also encounter :

xl xh       : x is a word, read LSB first (l=low / h=hi)

Ll Lh Hl Hh : read a double word, LSB first.

This is Pascal BOURON's notation (thanks !).

- No type is signed unless stated. (byte,word,double word)

- Due to the fact that the Motorola MC68000 processor uses the big endian

format, you must read integers from left to right, LSB first (Least

Significant Byte). Example :

+10        4       12 51 78 A3 (double word)

You will read the double word A3785112 at offset 10 (hexa).

Remember that Intel's processors (ix86) use little endian format : you

read integers from left to right, MSB first (Most Significant Byte). Thus

12 51 78 A3 is the double word 125178A3.

LSB (TI92) or MSB (PC) first will be specified for integers longer than

a byte.

Practically, you will only find big endian format (MC68000, LSB first)

integers in TI92's protocol. Little endian format only appears on PC's

archive files.

- These informations are the result of long researches (since 02/95!), and

have been completed with and compared to TI-PROT.TXT by Pascal BOURON

(bouron@ENS-Cachan.fr). Hardware communication routines by Ben EATER

(themouse@wam.umd.edu) and George NACHMAN (ltduck@wam.umd.edu).

You can find other informations from David ELLSWORTH (davidells@aol.com),

the author of FARGO (TI92 assembly support), Dan EBLE and Rob TAYLOR

(see http://www.ticalc.org).

- General consideration on TI92's behaviour (reliable for ROM 1.3) :

* unless stated, all communications run well when the TI92 is in its

HOME screen. Variable transfer won't work if a dialogue is open (the

TI92 will send 'not ready' to any attempt).

* if the TI92 is switched off, it will turn on when a byte is send thru

the link port. The TI92 needs about 800 ms to wake up (delay to reach

the 'ready' state).

* the TI92 auto-power off feature will turn off the calc within 1 or 2

minutes when the TI92 has been woken up by the link port. Normal

activity auto-power off needs 6 idle minutes.

* with "home made" cables, the TI92 seems to freeze sometimes. This is

due to LPT1 idle state (0) not corresponding to TI92 idle state (lines

to 1).

Win'95 always reset LPT1 to 0 when closing a DOS session (Win3.1 does

not). So be careful, and remember to unplug your TI92 when you are

finished (or else the batteries won't cope).

Note that the TI92 needs about 800 ms to recover from the "frozen"

state (that is to be in a 'ready state').

����������Ŀ

� HARDWARE �

������������

���Ŀ

� 1 � HOME MADE CABLE

���������������������

This cable works on a parallel port of a PC computer. It should actually

work with any computer having a standard parallel port.

Components required :

- a 25 pins male D-sub

- a 2.5 mm stereo jack

- 2 wires shielded cable (<1.5 m)

- 2 silicon diodes (supporting some mA, 0.6V threshold, like 1N4148)

\�����������������������������������������������������/

\ .   .   .   .   .   .   .   .   .   .   .   .   . /

\  .   .   .   .   .   .   .   .   .   .   .   .  /   D-sub 25pin male

\�����������������������������������������������/

13             3      12             2   22        Pin number

�             �       �             �    �

� ���������Ŀ �       � ���������Ŀ �    �

�Ĵ    D  � ���       �Ĵ    D  � ���    �        Diodes

� �����������         � �����������      �

�                     �                  �

�                     �                  �

�                     �                  �

Head                  Ring              Shield

Head  Ring  Shield

�     �      �

�     �      �    ��������

��������������������

����   ���  �������

�����   ���  �������                          2.5 mm Jack

����   ���  �������

������������������Ŀ

��������

Parallel port pin-out :

2  D0   data bit 0

3  D1   data bit 1

12  PO   Paper Out

13  OL   On Line

22  GND  Ground

Be carefull when purchasing the 2.5 mm jack plug. They often have

slightly different shapes that could lead to the following problems.

If the pin is too tight, you will have bad connections. But if the pin

has a too big 'head', you are likely to damage your I/O port. Further

use of TI's cable (with cylindrical pin) could be impossible.

���Ŀ

� 2 � HARDWARE PROTOCOL

�����������������������

The TI92 calculator doesn't use the standard RS232 serial link protocol.

Describing the complete hardware protocol goes below the scope of this file.

If you use a home-made parallel port link cable, the following pseudo-code

will be convenient for a PC:

(thanks to George NACHMAN and Ben EATER)

- InPort and OutPort refer to the two different registers of the PIO

controller. For a PC, OutPort=LPTx base address (data register),

Inport=OutPort+001 (status register). Pins D0 and D1 are used for PC

output, pins OL (On Line) and PO (Paper Out) are used for PC input.

OutPort=$01 (bit 0, data register)  -> D0 set to +5V

OutPort=$02 (bit 1, data register)  -> D1 set to +5V

InPort =$10 (bit 4, status register)-> OL set to +5V

Inport =$20 (bit 5, status register)-> PO set to +5V

- The Wait() instruction waits for a boolean to be true or a constant

delay to be up. When a problem occurs, the execution should not be

stuck in a Wait(), waiting for a never coming InPort value.

Procedure Send (x : byte)

� Byte n

� For n<-0 To 7                 ;send low to hi bit of x

�   OutPort<-$03                ;assume port is idle

�   If (x And 2^n)=0

�     OutPort<-$01              ; bit n of x is cleared

�     Wait (InPort And $20=0)

�     OutPort<-$03

�     Wait (InPort And $20=$20)

�   Else

�     OutPort<-$02              ; bit n of x is set

�     Wait (InPort And $10=0)

�     OutPort<-$03

�     Wait (InPort And $10=$10)

�   EndIf

� EndFor

End Send()

Function Get : Byte

� Byte i,n,x

� x<-0

� For n<-0 To 7                           ;get low to hi bit of x

�   Wait ((i<-InPort And $30)<>$30)       ;C-like code !

�   If i=$10

�     x<-x+2^n                            ;bit n of x is set

�     OutPort<-$01

�     Wait ((InPort And $20)=$20)

�   Else

�     OutPort<-$02                        ;bit n of x is cleared

�     Wait ((InPort And $10)=$10)

�   EndIf

�   OutPort<-$03                          ;assume port is idle

� EndFor

� Return (x)

End Send

These routines should be working on other platforms than PC... I would

be pleased hearing that. I may also try them on a HP Apollo/9000 (Unix

platform), but I don't think a lot of people would be interested in a

Unix version...

I reached a 6 Kb/s transfer rate with an assembler version of these

routines, but they were unreliable on fast computers. A safe way to code

them is to code them slow.

����������Ŀ

� SOFTWARE �

������������

���Ŀ

� 1 � TI PACKETS

����������������

The TI92 calculator like other TI calcs doesn't communicate in raw mode,

that is sending datas directly to the port. Instead, the protocol consists

of a dialogue between the computer and the calc. This dialogue consists of

words called 'packets'. Physically, a packet cannot be less than 4 bytes,

and has a standard format :

HEADER

+0     1    Sender

+1     1    Type

+2     2    info (often n = the packet size)

BODY          (some packet types only)

+4     n    data

CHECKSUM      (some packet types only)

+4+n   2    checksum

Sender :

09 = Computer

89 = TI92

Type :

06 = Variable header

size=info (LSB first) (7 to 23 bytes)

09 = Wait state

size=0

info=0 or -1

15 = Variable data

size=info (LSB first)

56 = Busy state

size=0

info=0 (sender ready)

info=1 (sender busy)

68 = Check state

size=0

info=0

6D = Screen dump

size=0

info=0

78 = Continue

size=0

info=0

87 = Remote control

size=0

info=key code (LSB first) (see TI92 manual)

92 = End of transmission

size=0

info=0

A2 = Request (variable or directory)

size=info (LSB first)

���Ŀ

� 2 � PACKETS DESCRIPTION

�������������������������

xx stands for the sender, equals 09 for PC->TI92 transfers, 89 for

TI92->PC. Some packets are unidirectionnal (only PC->TI92 for 6D/screen

dump, 87/remote control, A2/request).

06 (variable header)

��������������������

HEADER     xx 06 lo hi

BODY       Ll Lh Hl Hh ty le c1 c2 ...

CHECKSUM   sl sh

lo hi       = BODY size (7..23d bytes) (word)

Ll Lh Hl Hh = variable size (long word)

ty          = variable type (see table below)

le          = length of the variable name string (1..17 chars)

c1 c2 ...   = variable name string

sl sh       = checksum of BODY (word)

Valid TI92 variables types are :

00   EXPR          0E   FIG

04   LIST          10   PIC

06   MAT           12   PGRM

0A   DATA          13   FUNC

0B   TEXT          14   MAC

0C   STR           19   DIR

0D   GDB           1F   FOLD

The 'DIR' type is only used for the TI92's directory request. Do not

mix up with 'FOLD' which is a TI92 folder. (see section 5) The TI92

also uses a pseudo-variable for backups, type 1D.

The name of a variable is between 1 and 17d bytes long. A directory

path can be added to the name of the variable, using char '\'. Both

names and directories must not exceed 8 chars. Examples :

'x'

'var1'

'main\example'

'dir_maxx\var_maxx'

When the TI92 receives a single variable name "varxxxxx", it will

store it on its current folder, and overwrite it without any warning

if the variable already exists.

If the TI92 receives a variable name with its path "pathxxx\varxxxx",

it will try to receive it to the "pathxxxx" folder. If the folder does

not exist, it will be immediatly created. If the variable already

exists, it will be overwritten without any warning.

(These two paragraphs apply to the direct transfer protocol, working

while the TI92 is in its HOME page. You will get erasing confirmations

if you use the Var-Link.)

Variable type 1F is special : this is the description of a TI92

folder. In this case, the variable name string is a 1 to 8 chars

string, and the variable size refers to the number of variable held

in this folder.

09 (Wait state)

���������������

HEADER     xx 09 00 00

BODY       none

CHECKSUM   none

The sender means to the receiver it is waiting for data.

15 (variable data)

������������������

HEADER     xx 15 lo hi

BODY       ?? ?? ?? ...

CHECKSUM   sl sh

lo hi = BODY size (word)

sl sh = checksum of BODY (word)

The body of variable data packets depends on the variable type.

You can read the description of some variable data packet bodies in

section 3.

56 (Busy state)

���������������

HEADER     xx 56 00 st

BODY       none

CHECKSUM   none

st = state byte

If st=0 then the sender (xx) is ready for transfer. If st<>0, the

sender is not ready to accept data (st=1 usually).

68 (State request)

������������������

HEADER     xx 68 00 00

BODY       none

CHECKSUM   none

This packet is a 'command' that waits for a 'busy state packet' (56)

and thus check if the receiver is available for transfer.

6D (screen dump)

����������������

HEADER     09 6D 00 00

BODY       ?? ?? ?? ...

CHECKSUM   sl sh

lo hi = BODY size (word)

sl sh = checksum of BODY (word)

The TI92 can be asked a screen backup at any time. It will answer

a data packet (type 15), whose body contains a 240*128d bitmap (3840d

bytes). See section 6.

87 (remote control)

�������������������

HEADER     09 87 lo hi

BODY       none

CHECKSUM   none

lo hi = key code (word)

The remote control feature perfectly simulates a key press on TI92's

keyboard. See section 8.

92 (end of transmission)

������������������������

HEADER     xx 92 00 00

BODY       none

CHECKSUM   none

This is a command used at the end of a transfer dialogue, to release

the receiver device.

A2 (variable/directory request)

�������������������������������

HEADER     09 A2 lo hi

BODY       00 00 00 00 ty le c1 c2 ...

CHECKSUM   sl sh

lo hi     = BODY size (6..23 bytes) (word)

ty        = variable type (see table at packet 06)

le        = length of the variable name string (0..17 chars)

c1 c2 ... = variable name string

sl sh = checksum of BODY (word)

This command packet asks the TI92 to send a variable or its complete

directory. See section 4 and 5 for more details.

���Ŀ

� 3 � VARIABLE DATA DESCRIPTION

�������������������������������

This section describes the BODY of the 'variable data packet' (15) send

in a TI92->PC transfer. The storage of information is of course dependent

on the type of the variable.

TEXT

+0       2      0  0

+2       4      variable size (TI Var-link size-2) = n+4

+6       2      position of cursor (in characters)

+8       n      characters (see remarks)

+8+n     2      0 E0

Remarks:

-A line always begins with a #32 (blank) character.

-A line ends with a #13 (RC) character, except for the last line.

... (to be continued)

���Ŀ

� 4 � RETRIEVE/SEND VARIABLE

����������������������������

The protocol is described by the following dialogue, where ss is sender,

and rr is receiver (09 or 92). (step 1 and 2 are optionnal)

The SEND protocol (PC->TI92) is handled by the I/O port interrupt routine

of the TI92, which can only be triggered when it is in its HOME screen. The

Var-Link uses this dialogue from step 3, but needs an operator to set the

TI92 in receive mode.

The RETRIEVE protocol (TI92->PC) will be identical, but this time the TI92

will trigger it by sending a 'variable header packet' (06). (step 3)

1- ss 68 00 00     is receiver ready ?

2- rr 56 00 xx     xx=0  -> ready

xx<>0 -> busy, go step 1

3- ss 06 lo hi     send variable header

?? ?? ?? ...

4- rr 56 00 xx     xx=0  -> ready

xx<>0 -> busy, give up

5- rr 09 00 00     waiting for data

6- ss 56 00 00     sender ready, data coming up

7- ss 15 lo hi     variable data

?? ?? ?? ...

8- rr 56 00 xx     xx=0  -> all right

xx<>0 -> busy, give up

9- ss 92 00 00     end of transfer

10-rr 56 00 xx     xx=0  -> ended

xx<>0 -> busy, give up

Hopefully the TI92 supports a 'request variable' function, that is really

simple. This will only work if the TI92 is in its HOME screen, with no open

dialogue box, or else you will get a 'busy state' from the TI92.

To use the request, add step 2a and 2b after step 2 of the preceding

dialogue with :

2a- 09 A2 lo hi     computer requests a variable

?? ?? ?? ...    see section 2

2b- 89 56 00 xx

If the variable requested does not exist, the dialogue stop at step 2b,

the calc sending a 'not ready state' (56 with info xx<>0).

���Ŀ

� 5 � TI92 DIRECTORY

��������������������

The TI92 directory request is a particular case of the request function.

You have to request for a variable whose name string is a null string, and

whose type is 19 (DIR).

1- 09 A2 06 00         HEADER   PC requests TI92's directory

00 00 00 00 19 00   BODY

19 00               CHECKSUM

2- 89 56 00 xx         xx=0  -> TI92 ready

xx<>0 -> TI92 busy, go step 1 or give up

3- 89 06 lo hi         HEADER   variable header (folder description)

?? ?? ?? ...        BODY

sl sh               CHECKSUM

At step 3, the TI92 sends in a 'variable header packet' the description

of a folder(???). Then follows this recurrent dialogue :

REPEAT

1- 09 56 00 00         computer ready

2- 09 09 FF FF         computer waiting

3- 89 56 00 xx         xx=0  -> TI92 ready

xx<>0 -> TI92 busy, give up

4- 89 15 lo hi         HEADER   variable/folder description

00 00 00 00

c1 .. c8            variable/folder name string (8 chars)

ty                  variable type

st                  variable state (0=normal, 1=edit)

Ll Lh Hl Hh         variable size

(number of variables if variable type is folder/1F)

sl sh               CHECKSUM

5- 09 56 00 00         Computer ready

6- 89 xx 00 00         Continue (xx=78) or end of transmission (xx=92)

UNTIL xx<>78             xx should be 92 to terminate

The TI92 will sequentially send :

- a folder 'foldxxxx' (variable type 1F, size = number of variables held)

- variables 'varxxxxx' belonging to 'foldxxxx', if it is not empty.

- another folder, then its content...

Note that the TI92 will send folder and variables in alphabetical order.

���Ŀ

� 6 � SCREEN DUMP

�����������������

The TI92 can be asked for a screen dump at any time, even when 'BUSY'

is highlited on the screen. It will stop everything and send the dump

thru the I/O port. This operation should take a few seconds (1 second

mini). Here is the dialogue :

1- 09 6D 00 00     PC requests screen dump

2- 89 56 00 xx     xx=0  -> TI92 ready

xx<>0 -> TI92 busy, go step 1 or give up

3- 89 15 lo hi     HEADER data packet

?? ?? ?? ...    BODY   240*128d bitmap (3840d bytes)

sl sh           CHECKSUM

4- 09 56 00 00     PC received (ready)

lo hi = 3840d bytes (240*128d bits)

���Ŀ

� 7 � BACKUPS

�������������

Backups consist in a partial dump of TI92's memory. They can be about

from 38 Kb to 100 Kb. The TI92 can receive or send a backup at any time.

If a send (to TI) backup operation fails in its middle, the TI92 will

reset and restart a new (clean) session. (FARGO installation is an

exception).

Receiving the backup, the dialogue begins like a variable request :

1- 09 A2 11 00       request

00 00 00 00 1D 0B 6D 61 69 6E 5C 62 61 63 6B 75 70  "main\backup"

type 1D, size 0

9F 04             checksum

2- 89 56 00 xx       xx=0 (ready), busy else

REPEAT

3- 89 06 09 00     variable header

Bl Bh 00 00 1D 03 31 2E 33  block size = B bytes block

ROM        = "1.3"

type       = 1D

B6 00           checksum

4- 09 56 00 00     PC OK

5- 09 09 Bl Bh     waiting for BhBl bytes

6- 89 56 00 xx     xx=0 (ready), busy else

7- 89 15 Dl Dh     data packet, DhDl=BhBl+4 bytes

?? ?? ?? ...

Sl Sh           checksum

8- 09 56 00 00     PC OK

END REPEAT

9-  89 92 00 00      TI92 end of transmission

10- 09 56 00 00      PC OK

Some explanation :

You never know the total size of the backup until it is finished ! The

best way to detect the end of the REPEAT ... END REPEAT is to check for

the 89 92 00 00 (end of transmission) packet received at step 3 instead

of the packet header.

Data packets are 4 bytes longer than the announced size in the variable

header packet. That's the way TI92 send variables. To store the backup in

a file, just leave the 4 first bytes of each block, and concatenate them

into one block.

The TI92 usually send backups in 1024d bytes blocks, unless the last

block is undetermined since the backup size is not likely to be a multiple

of 1024d !

And now, how to send a backup. If you stop the conversation too long

(about 1.5 sec. considered as 'time up' by the TI92), the TI92 will auto-

maticly reset (and that's handy). Here is the dialogue :

1- 09 06 05 00       variable header

Ll Lh Hl Hh 1D    backup size, type 1D

Sl Sh             checksum

2- 89 56 00 xx       xx=0 (ready), busy else

REPEAT

3- 09 06 05 00     variable header

00 00 00 00 1D  type 1D

1D 00           checksum

4- 89 56 00 xx     xx=0 (ready), busy else

5- 89 09 00 00     waiting for block

6- 09 56 00 00     PC OK

7- 09 15 Bl Bh     BhBl = block size

?? ?? ?? ...    block data

Sl Sh           checksum

8- 89 56 00 xx     xx=0 (ready), busy else

END REPEAT

9 - 09 92 00 00      end of transmission

Some explanation :

The transfer should be done with 1024d blocks like a lot of ROMs do when

a TI92 send a backup. These blocks can be taken rawly in a backup file.

Have a look at FORMATS.DOC for more technical information.

If the transfer succeed, the TI92 will reset (blue screen) and restart

the session you backed up before. That's all...

���Ŀ

� 8 � REMOTE CONTROL

��������������������

There are two ways to remotely control your TI92.

You can put your TI92 in a special mode. From the Home screen, press

F5 + Diamond + '(' + 'R' (one after each other). The TI92 will now

immediatly convert a word received from the link port as a key stroke.

The TI92 will send the FF byte after each word. (See TI92 manual for the

key codes)

Output is also redirected to the link port. Any result (Enter key pressed)

is send in ASCII code with a terminal FF byte.

To reset the TI92 back to normal mode, send the magic keyword thru the

port : F5 + Diamond + '(' + 'R'.

You can use the TI92 in its normal mode. Send a RC packet :

09 87 ll hh    where hhll is the key code (see TI92 manual)

The TI92 will answer for each code sent :

89 56 00 xx    xx=0 (ready), busy else

This kind of remote control totally simulates the strike of a key. Any

result will be displayed on TI92's screen just as if a user was in front

of it.

- End of PROTOCOL.DOC -